UNIVERSITY CAMPUS NETWORK

DESIGN AND IMPLEMENTATION

Cisco Packet Tracer Project Documentation

Submitted By

Sarib Shahid

Course: Computer Networks

Software: Cisco Packet Tracer

# 1.  Project Overview

This project designs and implements a university campus network using Cisco Packet Tracer. The network is divided into the following departments:

- Administration

- Academic

- Library

- Hostel

- Server Farm

The network provides the following services and features:

- VLAN Segmentation

- Inter-VLAN Routing

- Dynamic Routing Protocols

- DHCP

- DNS

- HTTP / HTTPS

- FTP Services

- Email Services

- Internet Connectivity Simulation

# 2.  Objectives

- Create a hierarchical campus network.

- Implement VLAN-based segmentation.

- Configure Router-on-a-Stick inter-VLAN routing.

- Implement RIP, OSPF, and EIGRP routing protocols.

- Provide automatic IP addressing through DHCP.

- Implement DNS services.

- Host Web, FTP, and Email servers.

- Enable communication between all departments.

- Simulate Internet connectivity.

# 3.  Network Topology

## 3.1  Routers

| Router | Purpose |
| --- | --- |
| R1 | Internet Edge Router |
| R2 | Administration Router |
| R3 | Academic Router |
| R4 | Library Router |
| R5 | Hostel Router |

## 3.2  Switches

| Switch | Purpose |
| --- | --- |
| CORE-SW | Core Switch |
| SW-ADMIN | Administration Distribution Switch |
| SW-V10 | Admin VLAN 10 Access Switch |
| SW-V20 | Admin VLAN 20 Access Switch |
| SW-ACADEMIC | Academic Distribution Switch |
| SW-V30 | Academic VLAN 30 Access Switch |
| SW-V40 | Academic VLAN 40 Access Switch |
| SW-LIBRARY | Library Distribution Switch |
| SW-V50 | Library Access Switch |
| SW-HOSTEL | Hostel Distribution Switch |
| SW-V60 | Hostel Access Switch |
| SW-SERVERS | Server Switch |

# 4.  VLAN Design

| VLAN ID | Name | Network |
| --- | --- | --- |
| 10 | HR | 192.168.10.0/24 |
| 20 | Finance | 192.168.20.0/24 |
| 30 | Faculty | 192.168.30.0/24 |
| 40 | Students | 192.168.40.0/24 |
| 50 | Library | 192.168.50.0/24 |
| 60 | Hostel | 192.168.60.0/24 |
| 70 | Servers | 192.168.70.0/24 |
| 99 | Management | 192.168.99.0/24 |

# 5.  IP Addressing Scheme

## 5.1  Core Links

| Connection | Network |
| --- | --- |
| CORE ↔ R2 (Administration) | 172.16.2.0/30 |
| CORE ↔ R3 (Academic) | 172.17.3.0/30 |
| CORE ↔ R4 (Library) | 172.18.4.0/30 |
| CORE ↔ R5 (Hostel) | 172.19.5.0/30 |
| CORE ↔ R1 (Internet Edge) | 10.1.1.0/30 |

# 6.  Inter-VLAN Routing

Router-on-a-Stick was implemented on routers R2, R3, R4, and R5. Subinterfaces were configured using 802.1Q encapsulation:

encapsulation dot1Q <VLAN-ID>

Each VLAN gateway was configured on the corresponding router subinterface. Example gateways:

| VLAN | Gateway IP |
| --- | --- |
| VLAN 10 (HR) | 192.168.10.1 |
| VLAN 20 (Finance) | 192.168.20.1 |
| VLAN 30 (Faculty) | 192.168.30.1 |
| VLAN 40 (Students) | 192.168.40.1 |
| VLAN 50 (Library) | 192.168.50.1 |
| VLAN 60 (Hostel) | 192.168.60.1 |

# 7.  Dynamic Routing Configuration

Different routing protocols were implemented across different departments to demonstrate protocol diversity.

## 7.1  RIP Version 2

Configured between: CORE-SW and R2 (Administration)

Advertised Network

172.16.0.0

192.168.10.0

192.168.20.0

## 7.2  OSPF — Area 0

Configured between: CORE-SW and R3 (Academic)

Advertised Network

172.17.3.0

192.168.30.0

192.168.40.0

## 7.3  EIGRP — AS 100

Configured between: CORE-SW, R4 (Library), and R5 (Hostel)

Advertised Network

172.18.4.0

172.19.5.0

192.168.50.0

192.168.60.0

# 8.  DHCP Configuration

A dedicated DHCP Server was configured with the following IP address:

DHCP Server IP: 192.168.70.10

DHCP pools were created for VLANs 10, 20, 30, 40, 50, and 60. Each pool includes:

- Gateway (Default Router)

- DNS Server: 192.168.70.20

- Starting IP Address

| Pool Name | Gateway | Start IP |
| --- | --- | --- |
| VLAN10 | 192.168.10.1 | 192.168.10.100 |
| VLAN20 | 192.168.20.1 | 192.168.20.100 |
| VLAN30 | 192.168.30.1 | 192.168.30.100 |
| VLAN40 | 192.168.40.1 | 192.168.40.100 |
| VLAN50 | 192.168.50.1 | 192.168.50.100 |
| VLAN60 | 192.168.60.1 | 192.168.60.100 |

All department routers were configured with the DHCP relay command:

ip helper-address 192.168.70.10

# 9.  DNS Configuration

DNS Server IP: 192.168.70.20

| Hostname | IP Address |
| --- | --- |
| www.campus.local | 192.168.70.30 |
| ftp.campus.local | 192.168.70.40 |
| mail.campus.local | 192.168.70.50 |
| dhcp.campus.local | 192.168.70.10 |

# 10.  Web Server Configuration

| Parameter | Value |
| --- | --- |
| Server IP | 192.168.70.30 |
| Services | HTTP and HTTPS |
| URL | http://www.campus.local |
| Content | University Campus Network — Welcome Page |

# 11.  FTP Server Configuration

| Parameter | Value |
| --- | --- |
| Server IP | 192.168.70.40 |
| Username | student |
| Password | cisco123 |
| Permissions | Read, Write, Delete, Rename |

# 12.  Email Server Configuration

| Parameter | Value |
| --- | --- |
| Server IP | 192.168.70.50 |
| Domain | campus.local |
| Services | SMTP and POP3 |

## 12.1  Email Accounts

| User | Email Address |
| --- | --- |
| admin | admin@campus.local |
| student | student@campus.local |
| faculty | faculty@campus.local |

# 13.  Internet Connectivity

R1 acts as the Internet Edge Router. A loopback interface (8.8.8.8) is used to simulate an Internet destination.

Default route configured on the Core switch:

ip route 0.0.0.0 0.0.0.0 10.1.1.1

All VLANs can successfully reach the simulated Internet address (8.8.8.8).

# 14.  Testing and Verification

## 14.1  VLAN Connectivity

✔  VLAN 10 ↔ VLAN 20

✔  VLAN 30 ↔ VLAN 40

✔  VLAN 50 ↔ VLAN 60

## 14.2  Inter-Department Connectivity

✔  Administration ↔ Academic

✔  Administration ↔ Library

✔  Administration ↔ Hostel

✔  Academic ↔ Library

✔  Academic ↔ Hostel

✔  Library ↔ Hostel

## 14.3  Service Tests

| Service | Test Command / Action | Result |
| --- | --- | --- |
| DHCP | PC set to automatic — IP assigned | ✔ Pass |
| DNS | ping www.campus.local | ✔ Pass |
| Web (HTTP) | http://www.campus.local | ✔ Pass |
| FTP | ftp ftp.campus.local | ✔ Pass |
| Email (SMTP) | admin@campus.local → student@campus.local | ✔ Pass |
| Internet | ping 8.8.8.8 from all VLANs | ✔ Pass |

# 15.  Conclusion

The University Campus Network was successfully designed and implemented using Cisco Packet Tracer. The project achieved all stated objectives:

- VLAN Segmentation across all departments

- Inter-VLAN Routing via Router-on-a-Stick

- Multi-protocol dynamic routing (RIP, OSPF, EIGRP)

- Centralized DHCP with per-VLAN address pools

- DNS with fully qualified domain name resolution

- Web, FTP, and Email server deployment

- Simulated Internet connectivity from all VLANs

The network provides efficient, organized communication between all campus departments while maintaining scalability for future expansion.